Windows 2000 IR Setup Procedure for ACTiSYS ACT-IR2000B/L

C) Copyright 1998-2000, ACTiSYS Corp.

Note:

- Do not install Windows 95 Infrared Driver Ver 3.0 under Windows 2000, it will not work!!!

- Connect ACT-IR2000L adapter to the Mini DIN 8 IrDA port of the IR2000B.  In the ROM PCI/ISA BIOS, please disable the COM2 (for IR2000B only).

- If you do not have the driver floppy, download the file ns2k111r.inf from the ACTiSYS web site (http://www.actisys.com/files) and save it to a floppy.

Hardware Setup:

1.   Go to "Control Panel" and double click on "Add / Remove Hardware".

- On the Window, click "Next".

- On the following Window, choose "Add/Troubleshoot a Device" and click "Next".

- On the next Window, highlight "Add New Hardware", which is the 1-st line of Device list, and click "Next"

- Next, choose "No, I want to select the Hardware from a list" and click "Next".

- On the next window, select "Infrared devices" and click "Next".

- Click on "Have Disk".

- Insert the floppy that contains the file ns2k111r.inf file (See note above).  A new window will pop up and select "Browse". (NOTE: make sure that you are browsing the A: drive).

- Another window will pop up and highlight "NSCIRDA" and click on "OK".

- A new window will pop up and under Infrared Devices , the screen will show ACTiSYS IR 2000B FAST Infrared Port.  Highlight "ACTiSYS IR 2000B FAST IR Port" and click "Next".

- Click "Next" again.

- Click "Finish".

- Restart Computer.

- After reboot, go to "Control Panel" and double click "System".

- On the next window select "Hardware".

- Select "Device Manager".

- Under Infrared Devices, an exclamation mark should appear next to the ACTiSYS IR 2000B Fast Infrared Port. Double click "ACTiSYS IR Fast Infrared Port".

- Under Device Status, a 'Code 12' should be seen.  Select "Resources" on the top of the window and select "Set Configuration Manually".

- In the Configuration Window, highlight the first "Input/Output Range" and click on "Change Settings".  A new window will pop up and set the Value to "02F8-02FF" and click on "OK".  The change that you made should be visible in the Configuration Window.

- In the Configuration Window, highlight the second "Input/Output Range" and click on "Change Settings.  A new window will pop up and set the Value to "0150-0151" and click on "OK".  The change should be visible in the Configuration window.

- In the Configuration Window, highlight "Interrupt Request" and click on "Change Settings.  A new window will pop up and set the Value to "03" and click on "OK".  The change should be visible in the Configuration window.

- In the Configuration Window, highlight "Direct Memory Access" and click on "Change Settings.  A new window will pop up and set the Value to "00" and click on "OK".  The change should be visible in the Configuration window.

- If "?" or "???" still appears in the Configuration Window, click on the correct heading and "Change Setting" to the proper number listed above in steps 19-22 otherwise go to step 24.

Note: If a number already appears in the Value of the Change Setting window, retype the number before clicking on "OK".

- In the Configuration Window, click on "OK"

- You are now finished installing the ACT-IR2000B/L.  Restart your computer.