# Windows 98 IR setup for ACT-IR2000B + IR2000L(or IR2000LC)

Caution: Do not install Infrared Driver Ver 3.0 under Windows98, it will not work at all.

Connect ACT-IR2000L adapter to the Mini DIN 8 IrDA port of IR2000B.  In PC BIOS, please disable COM2 (for IR2000B only).

- Go to “Control Panel” and double

click on “Add New Hardware”.

- On the following screen, click on “Next” twice, then select “No, the device isn’t in the list”. And at the next screen, click on “Next”.

- Select “No I want to select

Hardware from the list”,

- Scroll down the menu and

highlight “Infrared Devices”,

then click on “Next” twice.

- Under “Manufacturers” Select

“(National Semiconductor)”,

and then in “Infrared Devices”

Select “(NSC-PC87108 Fast

Infrared Port)”.

Click on “OK”.

- At “Infrared Transceiver A”, Select  “HP-HSDL-1100/2100”for IR2000L-PH,

IR2000LC-PH & IR2000L-HP*.

“Plug and Play”for IR2000L-PI,

IR2000LC-PI & IR2000L-IBM*.

Then click on “Next”.

At “Infrared Transceiver B”, Select

“None”. Then click on “Next”.

- At “ConfigIoBaseAddress”

Select “0x150”

Click on  “Next”.

- Follow the screen prompts until it asks you what default ports to use.  Select “Use default ports (recommended)”.  Click on “Next”.

- When the process is completed, please go to “Control Panel” and double click on “Network”.  Select “NSC-PC87108 Fast Infrared Port”. Click on “Properties”.

- In NSC-PC87108 Fast Infrared Port Properties Go to Resources and change I/O address range: 2E8-2EF,

IRQ: 3,

DMA: 0

Click “Close” and “OK”

- Please reboot your computer.

- Please refer to ACT-IR 210L/2000L/2000B User’s Manual, “Page 13” to continue installation.

- * Module type located on the shipping invoice.

(Ver.0.1, 9-03-98)